Digitally signed

CVETKOVIC MIODRAG

Certificate issuer:

Ministry of the Interior of the Republic of Serbia

09 September .2024 13:24:51

Reg. No.		TIN:		Activity code:		Current account:

__________________________________________________________________________________________

1 – ARCHITECTURE PROJECT

Investor:	Municipality Merosina

17 Cara Lazara St. Merosina

Facility:	Business facility Po+P (basement+ground floor)

Cad. plot П. 1358/2, C.M. Oblacina (facility No. 5)

Type of technical documentation:	IDP- Conceptual design

For construction / execution of works:	Reconstruction

Project designer:	“STUDIO ARHIJUG”

Obrenoviceva 46, TPC Kalca- AI 24, Nis

Responsible person/designer:	Miodrag Cvetkovic, MSc in Arch. Eng.

[Seal of the: Miodrag Cvetkovic design.

- Design studio for construction and

other types of structures –

STUDIO ARHIJUG - Nis]	Signature: [Signed]

Responsible designer:	Miodrag Cvetkovic, MSc in Arch. Eng.

License number:	300 R403 19

[Personal seal: Engineering

Chamber of Serbia –

Miodrag J. Cvetkovic,

MSc in Arch. Eng.

300 R403 19

Responsible designer]	Signature: [Signed]

Technical documentation No.:	41/24

Place and date:	Nis, September 2024

  - CONTENTS OF THE ARCHITECTURE PROJECT

| 1.1. | Front page |
| --- | --- |
| 1.2. | Contents |
| 1.3. | Decision on determining the responsible designer |
| 1.4. | Statement of the responsible designer |
| 1.5. | Textual documentation |
| 1.6. | Numerical documentation |
| 1.7. | Graphic documentation |

  - DECISION ON DETERMINING THE RESPONSIBLE DESIGNER

Based on the Law on Planning and Construction ("Official Gazette of RS," Nos. 72/09, 81/09—correction, 64/10 Constitutional Court decision, 24/11 and 121/12, 42/13—Constitutional Court decision, 50/2013—Constitutional Court decision, 98/2013—Constitutional Court decision, 132/14, 145/14, 83/14, 31/19, and 37/19—amended law, 9/2020, 52/2021, and 62/2023) and the provisions of the Rulebook on the Content, Method, and Procedure for the Preparation and Control of Technical Documentation According to the Class and Purpose of Buildings ("Official Gazette of RS," No. 96/2023) as:

R E S P O N S I B L E	D E S I G N E R

for the preparation of Project 1—Architecture, which is part of the Conceptual Design (IDP) for the reconstruction of the business facility Po+P (basement+ground floor) on Cad. Plot 1358/2 (Building No. 5), C.M. Merosina, the following is determined:

Miodrag Cvetkovic, MSc in Arch. Eng.	300 R403 19

Designer:	“STUDIO ARHIJUG”

Obrenoviceva 46, TPC Kalca- AI 24, Nis

Responsible person/designer:	Miodrag Cvetkovic, MSc in Arch. Eng.

[Seal of the: Miodrag Cvetkovic design.

- Design studio for construction and

other types of structures –

STUDIO ARHIJUG - Nis]	Signature: [Signed]

Technical documentation No.:	41/24

Place and date:	Nis, September 2024

1.4 STATEMENT OF THE ARCHITECTURE PROJECT RESPONSIBLE DESIGNER

The responsible designer of the architecture project, for the reconstruction of the business facility Po+P (basement+ground floor) on Cad. Plot 1358/2 (Facility No. 5), C.M. Merosina

Miodrag Cvetkovic, MSc in Arch. Eng.

I D E C L A R E

- that the project was prepared in accordance with the Law on Planning and Construction, regulations, standards and norms in the field of building construction and the rules of the profession;

- that during the development of the project, all prescribed and established measures and recommendations for the fulfillment of the basic requirements for the facility were respected and that the project was created in accordance with the measures and recommendations that prove the fulfillment of the basic requirements.

Responsible designer:	Miodrag Cvetkovic, MSc in Arch. Eng.

License number:	300 R403 19

[Personal seal: Engineering

Chamber of Serbia –

Miodrag J. Cvetkovic,

MSc in Arch. Eng.

300 R403 19

Responsible designer]	Signature: [Signed]

Technical documentation No.:	41/24

Place and date:	Nis, September 2024

  - TEXTUAL DOCUMENTATION

TECHNICAL DESCRIPTION

For the business facility, Cad. Plot 1358/2 C.M. Merosina (Facility No. 5)

For the needs of the investor, the Municipality of Merosina, located at 17 Cara Lazara Street, a project for the reconstruction of a business facility, with the structure of basement + ground floor, on Cad. plot no. 1358/2, cadastral municipality Oblacina (building no. 5), has been completed.

LOCATION

The facility is located on Cad. plot no. 1358/2, cadastral municipality Oblacina, within the territory of the Municipality of Merosina. The plot is situated in the II climate zone and the VII-VIII seismic zone.

GENERAL INFORMATION

The business facility in question is a freestanding structure, positioned in the southwestern part of the plot.

Access to the facility is from the southwest side. The entrance to the facility is also on the southwest side. The architectural design of the facility is detailed in the technical documentation.

A flat roof is planned, as was the case in the previous state.

EXISTING CONDITION

The building was built in a combined construction system with a flat roof. The size of the object does not change at all.

The building is registered in the real estate cadaster of the Republic Geodetic Institute as a building built before the adoption of regulations on building construction.

Project documentation for the existing building does not exist, and information about the building was obtained by recording the building on the spot, as well as from the representative of the investor - the Municipality of Merosina.

FUNCTIONAL DESIGN OF THE BUILDING

The project includes the reconstruction of the existing building within its current footprint and volume.

Reconstruction involves performing construction and other works on the existing structure, within its existing dimensions and volume, that affect the fulfillment of basic building requirements, alter the technological process, change the exterior appearance of the building, increase the number of functional units, or replace devices, equipment, and installations with increased capacity.

The reconstruction also includes interior work as well as some facade modifications and the replacement of windows and doors.

The building consists of Po+P (basement+ground floor). The total number of business units remains one (1).

Access to the building is from the southwest side, and the entrance is also located on the southwest side.

The layout and purpose of the building's rooms are provided in the project.

The layout and purpose of the rooms are detailed in the Conceptual Design.

The newly designed solution includes the following height characteristics:

- Leveling is in accordance with the courtyard.

The relative elevation ± 0.00 m corresponds to the absolute elevation +286.77

- Elevation of the final layer of the entrance platform:

- Ground floor entrance at +0.05 m

The ridge height remains the same at +6.25 m

CONSTRUCTION

EXISTING CONDITION

The building is constructed using a combined construction system with a flat roof.

According to the investor's statement, the foundation of the walls was completed on strip foundations of appropriate dimensions and depth.

The foundation pads are made of reinforced concrete, placed over a 10 cm thick layer of compacted natural gravel.

The foundation walls are constructed of reinforced concrete, poured into double-sided wooden formwork, and reinforced with horizontal and vertical reinforced concrete ties.

The ground floor slab is made of reinforced concrete, approximately 10-12 cm thick. The load-bearing walls of the building are constructed from masonry products in longitudinal mortar. The partition walls are also built from masonry products in longitudinal mortar.

The staircase in the building is made of reinforced concrete.

The terraces and canopies are also constructed from reinforced concrete.

The intermediate structure, or ceiling construction, is a semi-prestressed thin ribbed construction of the "Avramenko" type. A smaller portion of the structure is made as a monolithic reinforced concrete slab.

The roof is a flat roof.

A single chimney made of masonry products has been constructed on the building.

The interior walls and ceilings in the building are plastered and have undergone painting and finishing work; however, the plaster is now in very poor condition and must be completely removed.

The floor coverings in the building have been stripped.

The facade of the building has been finished, plastered, and painted with an appropriate color, but this plaster is also in very poor condition and needs to be entirely removed to ensure proper adhesion between the masonry walls and the future "demit" facade.

All existing reinforced concrete elements in the building will be retained.

NEWLY DESIGNED SOLUTION

The proposed works do not alter the building's structure, so they do not impact the stability of the building.

The reconstruction includes interior work as well as some modifications to the building's facade and the replacement of windows and doors. On the flat, accessible roof, a movable pergola and a metal railing will be added to create a seating area with a view of the lake for visitors waiting for their balloon ride. A list of the works is provided in the numerical documentation of the Architectural Project.

On the exterior walls, the existing extended plaster with a thickness of 3-4 cm will be removed, and in its place, the walls will be covered with appropriate thermal insulation of suitable thickness. This will ensure that the construction meets the requirements specified by the Regulation on the Energy Efficiency of Buildings.

Windows and doors will be made from materials that fully meet the requirements for thermal insulation of the building and will be installed accordingly.

Heating in the building is planned to be provided by devices that operate on electrical energy.

Rainwater drainage from the building's roof will be managed on the property itself. Greenery on the plot will be maintained at the same percentage as before.

Installations: Plumbing and sewage systems must be completed in accordance with all applicable legal regulations, rules, and standards.

Electrical installations must be completed in accordance with all applicable legal regulations, rules, and standards.

Telecommunication and signaling installations must be completed in accordance with all applicable legal regulations, rules, and standards.

The technical documentation includes measures to ensure that the building meets the following basic requirements:

Mechanical resistance and stability

Fire safety

Hygiene and health protection and environmental protection

Safety and accessibility in use

Noise protection

Economical energy use and heat conservation;

Mechanical resistance and stability ensure that the forces the building will be exposed to during construction and use do not cause:

Collapse of the entire building or parts of it

Deformations beyond the allowable level

Damage to other parts of the building, installations, or installed equipment due to significant deformations of the load-bearing structure

Fire safety ensures that in the event of a fire:

The required load-bearing capacity of the building's structure is maintained for a specified period

Users can evacuate the building or be rescued by other means.

The safety of individuals involved in firefighting and rescue operations is considered

Hygiene, health, and the environment ensure that during the construction, use, or removal of the building, the hygiene or health and safety of workers, users, or neighbors are not compromised.

Safety and accessibility during use ensure that the building does not pose risks of accidents or damage during operation or use.

Noise protection ensures that the noise to which users or neighbors are exposed is at a level that does not threaten their health and allows them to sleep, rest, and work under appropriate conditions.

Economical energy use and heat conservation, achieved through the selection of thermal insulation, heating, ventilation, lighting, and hot water preparation systems, ensure the lowest possible level of energy consumption while considering the users and the climatic conditions of the location.

[Personal seal: Engineering

Chamber of Serbia –

Miodrag J. Cvetkovic,

MSc in Arch. Eng.

300 R403 19

Responsible designer]		Signature: [Signed]

  - NUMERICAL DOCUMENTATION

Area Calculation:

| - Plot Area - Building footprint area | 46,463.00m2 182.00m2 |
| --- | --- |
| Gross area of the basement | 14.86m2 |
| Gross area of the ground floor | 182.00m2 |
| Net area of the basement | 8.79m2 |
| Net area of the ground floor | 144.03m2 |
| GROSS DEVELOPED BUILDING AREA |  |

- Ground floor	182.00m2 Total	182.00m2

УКУПНА НЕТО КОРИСНА ПОВРШИНА

- Basement	8.79m2

- Ground floor	144.03m2 Total	152.82m2

Building occupancy ratio

Horizontal projection of the building on the plot/plot area

182.00m2/ 46,463.00m2 x 100 = 0.39 %

Other structures on the plot: 4,69% 0.39% + 4.69% = 5.08%

Building coefficient

Gross developed area of the building/plot area

381.46m2 / 46,463.00m2 = 0.0039

Other structures on the plot: 0.0469 0.0039+0.0469=0.0508

[Personal seal: Engineering

Chamber of Serbia –

Miodrag J. Cvetkovic,

MSc in Arch. Eng.

300 R403 19

Responsible designer]		Signature: [Signed]

PRELIMINARY ESTIMATE OF THE BUILDING'S VALUE

Preliminary estimate of the building's value	15,200,000.00 dinars (RSD)

LIST OF WORKS DURING RECONSTRUCTION / PRELIMINARY ESTIMATE OF WORK

1.6.2. PRELIMINARY ESTIMATE OF WORK – PHASE I - PREPARATORY AND ROUGH WORKS

| POS. | POSITION DESCRIPTION | U.M | QUANT |  | UNIT PRICE |  | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- |

| II | PREPARATORY WORKS AND DEMOLITION WORKS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.1. | Installation of a construction site board |  |  |  |  |  |  |
|  | Calculation – flat-rate | flat r. | 1.00 | х |  | = | - |
| 2.2. | Cleaning and removal of waste in the ground floor and basement of the building – the building is currently in a ruined state, so it is necessary to first clean the waste present there. The waste should be transported to a landfill. |  |  |  |  |  |  |
|  | Calculation – flat-rate | flat r. | 1.00 | x |  | = | - |
| 2.3. | Removal of existing woodwork from the building |  |  |  |  |  |  |
|  | Calculation per unit | unit | 6.00 | x |  | = | - |
| 2.4. | Removal of internal plaster on walls that is in poor condition |  |  |  |  |  |  |
|  | Calculation per m² | m2 | 300.00 | x |  | = | - |
| 2.5. | Removal of internal plaster in poor condition from parts of the ceiling made of monolithic reinforced concrete slab |  |  |  |  |  |  |
|  | Calculation per m² | m2 | 30.00 | x |  | = | - |
| 2.6. | Removal of external plaster on the exterior walls, 3-4 cm thick, that is in poor condition (to ensure a good bond between the brick exterior walls and the thermal insulation cladding) |  |  |  |  |  |  |
|  | Calculation per m² | m2 | 240.00 | x |  | = | - |
| 2.7. | Cutting and removal to the landfill of high and medium deciduous vegetation that has grown in the atrium |  |  |  |  |  |  |
|  | Calculation – flat-rate | flat r. | 1.00 | x |  | = | - |
| 2.8. | Demolition of non-load-bearing internal walls made of bricks and masonry blocks, including parts of these walls to create new openings |  |  |  |  |  |  |
|  | Calculation per m² | m2 | 17.50 | x |  | = | - |
| 2.9. | Demolition of parts of load-bearing internal walls made of bricks to create new openings, and reinforcement with reinforced concrete elements if necessary |  |  |  |  |  |  |
|  | Calculation per m² | m2 | 47.50 | x |  | = | - |
| 2.10. | Removal of existing sewerage installations |  |  |  |  |  |  |
|  | Calculation – flat-rate | flat r. | 1.00 | x |  | = | - |
| 2.11. | Cleaning and removal of resulting rubble. Transport the rubble to a landfill. |  |  |  |  |  |  |
|  | Calculation – flat-rate | flat r. | 1.00 | x |  | = | - |
|  | Total preparation works: |  |  |  |  |  | - |
| III | CONCRETE WORK |  |  |  |  |  |  |
| 3.1. | Concrete work, specifically repairing the concrete floor in the atrium of the building |  |  |  |  |  |  |
|  | Calculation per m² | m² | 11.00 | x |  | = | - |
|  | Total concrete work: |  |  |  |  |  | - |

| IV | MASONRY WORKS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4.1. | Sealing certain openings of the building with brick blocks using an extended mortar mix of 1:3:9, in full accordance with the project plan |  |  |  |  |  |  |
|  | Calculation per m² | m² | 15.50 | x |  | = | - |
|  | Total masonry works: |  |  |  |  |  | - |
| V | VARIOUS MASONRY WORKS |  |  |  |  |  |  |
| 5.1. | Installation (procurement and installation) of horizontal waterproofing for the basement floor of the ground level |  |  |  |  |  |  |
|  | Calculation per m² | m² | 155.00 | x |  | = | - |
| 5.2. | Installation of vertical waterproofing on basement walls |  |  |  |  |  |  |
|  | Calculation per m² | m² | 40.00 | x |  | = | - |
| 5.3. | Installation of horizontal thermal insulation made of polystyrene, thickness 5 cm, on the ground floor |  |  |  |  |  |  |
|  | Calculation per m² | m² | 140.00 | x |  | = | - |
| 5.4. | Construction of a cement screed, 4-5 cm thick, in the basement and on the ground floor of the building |  |  |  |  |  |  |
|  | Calculation per m² | m² | 155.00 | x |  | = | - |
| 5.5. | Construction of protection for vertical waterproofing on basement walls with a 'cladding' wall—bricks 'on edge,' 6.5 cm thick |  |  |  |  |  |  |
|  | Calculation per m² | m² | 20.00 | x |  | = | - |
| 5.6. | Installation of a waterproofing membrane on the flat accessible roof of the building, with a transition at all parapets |  |  |  |  |  |  |
|  | Calculation per m² | m² | 190.00 | x |  | = | - |
| 5.7. | Installation of a thermal insulation layer made of polystyrene, 5 cm thick, on the flat accessible roof |  |  |  |  |  |  |
|  | Calculation per m² | m² | 146.00 | x |  | = | - |
| 5.8. | Construction of a sloping layer in the form of a cement screed, 4-10 cm thick, on the flat accessible roof of the building |  |  |  |  |  |  |
|  | Calculation per m² | m² | 146.00 | x |  | = | - |
| 5.9. | Construction of a cement screed, 4 cm thick, on the exterior—at the building’s porches and access staircases—as a base for anti-slip ceramic tiles |  |  |  |  |  |  |
|  | Calculation per m² | m² | 16.00 | x |  | = | - |
| 5.10. | Construction of partition walls for the future sanitary facility using drywall panels (Knauf system), 1.25 cm thick |  |  |  |  |  |  |
|  | Calculation per m² | m² | 29.00 | x |  | = | - |
| 5.11. | Construction of a ceiling using drywall panels (Knauf system), 1.25 cm thick, in parts of the ground floor ceiling where drywall panels are planned. |  |  |  |  |  |  |
|  | Calculation per m² | m² | 70.00 | x |  | = | - |
| 5.12. | Treatment of the ground floor ceiling structure of the 'Avramenko' type in areas where this ceiling structure remains visible. The treatment involves cleaning, i.e., 'wiping,' and possibly spraying with a specific material. |  |  |  |  |  |  |
|  | Calculation per m² | m² | 95.00 | x |  | = | - |
|  | Total various masonry works: |  |  |  |  |  | - |
| TOTAL: |  |  |  |  |  |  | - |
|  | RECAPITULATION: |  |  |  |  |  |  |
| II | PREPARATION WORK AND DEMOLITION WORK |  |  |  |  |  | - |
| III | CONCRETE WORKS |  |  |  |  |  | - |
| IV | MASONRY WORKS |  |  |  |  |  |  |
| V | VARIOUS MASONRY WORKS |  |  |  |  |  | - |
|  | T O T A L (din): |  |  |  |  |  | - |

1.6.2. PRELIMINARY ESTIMATE – PHASE II FINAL WORKS

| POS. | POSITION DESCRIPTION | U.M | QUANT |  | UNIT PRICE |  | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- |

| VI | CARPENTRY WORKS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6.1. | Procurement and installation of exterior double-leaf entry doors made of aluminum profiles, dimensions 217 x 210 + 45 cm. The right leaf is smaller, with dimensions 100 x 210 + 45 cm. Material, construction, details, and fittings are to be in accordance with the manufacturer's technology. |  |  |  |  |  |  |
|  | Calculation per unit | unit | 1.00 | x |  | = | - |
| 6.2. | Procurement and installation of a glass portal made of aluminum profiles, dimensions 241 x 255 cm, with a single-leaf door measuring 90 x 210 + 45 cm, facing the atrium. Material, construction, details, and fittings are to be in accordance with the manufacturer's technology. |  |  |  |  |  |  |
|  | Calculation per unit | unit | 1.00 | x |  | = | - |
| 6.3. | Procurement and installation of a fixed glass portal made of aluminum profiles, dimensions 200 x 240 cm. Material, construction, details, and fittings are to be in accordance with the manufacturer's technology. |  |  |  |  |  |  |
|  | Calculation per unit | unit | 2.00 | х |  | = | - |
| 6.4. | Procurement and installation of exterior folding glass balcony doors, dimensions 431 x 255 cm. Material, construction, details, and fittings are to be in accordance with the manufacturer's technology. |  |  |  |  |  |  |
|  | Calculation per unit | unit | 1.00 | x |  | = | - |
| 6.5. | Procurement and installation of an aluminum double-leaf window, dimensions 318 x 240 cm. One leaf is fixed, while the other is tilt-and-turn. Material, construction, details, and fittings are to be in accordance with the manufacturer's technology. |  |  |  |  |  |  |
|  | Calculation per unit | unit | 1.00 | х |  | = | - |
| 6.6. | Procurement and installation of a glass portal made of aluminum profiles, dimensions 405 x 300 cm, with an opening window. Material, construction, details, and fittings are to be in accordance with the manufacturer's technology. |  |  |  |  |  |  |
|  | Calculation per unit | unit | 1.00 | x |  | = | - |
| 6.7. | Procurement and installation of a glass portal made of aluminum profiles, dimensions 160 x 300 cm, with an opening window. Material, construction, details, and fittings are to be in accordance with the manufacturer's technology. |  |  |  |  |  |  |
|  | Calculation per unit | unit | 1.00 | х |  | = | - |
| 6.8. | Procurement and installation of an aluminum single-leaf window, dimensions 176 x 140 cm, with a tilt-and-turn function. Material, construction, details, and fittings are to be in accordance with the manufacturer's technology. |  |  |  |  |  |  |
|  | Calculation per unit | unit | 1.00 | х |  | = | - |
| 6.9. | Procurement and installation of aluminum single-leaf glass entry doors, dimensions 100 x 210 + 90 cm. Material, construction, details, and fittings are to be in accordance with the manufacturer's technology. |  |  |  |  |  |  |
|  | Calculation per unit | unit | 1.00 | х |  | = | - |
| 6.10. | Procurement and installation of garage doors in the prepared opening of 220 x 230 cm. The type of garage doors will be determined with the investor. Material, construction, details, and fittings are to be in accordance with the manufacturer's technology. |  |  |  |  |  |  |
|  | Calculation per unit | unit | 1.00 | x |  | = | - |

| 6.11. | Procurement and installation of internal aluminum doors, dimensions 80 x 210 cm, for sanitary areas. Fittings are to be in accordance with the manufacturer's technology . |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Calculation per unit | unit | 3.00 | х |  | = | - |
| 6.12. | Procurement and installation of internal glass doors made of aluminum profiles, dimensions 94 x 210 + 40 cm, for the office. Fittings are to be in accordance with the manufacturer's technology . |  |  |  |  |  |  |
|  | Calculation per unit | unit | 1.00 | x |  | = | - |
| 6.13. | Procurement and installation of internal aluminum doors, dimensions 90 x 210 + 40 cm, for the kitchenette. Fittings are to be in accordance with the manufacturer's technology . |  |  |  |  |  |  |
|  | Calculation per unit | unit | 1.00 | х |  | = | - |
| 6.14. | Procurement and installation of exterior aluminum doors for the room on the upper floor, dimensions 87 x 171 cm. Fittings are to be in accordance with the manufacturer's technology. |  |  |  |  |  |  |
|  | Calculation per unit | unit | 1.00 | x |  | = | - |
| 6.15. | Procurement and installation of a single-leaf aluminum window, dimensions 89 x 110 cm, with a tilt function. Material, construction, details, and fittings are to be in accordance with the manufacturer's technology . |  |  |  |  |  |  |
|  | Calculation per unit | unit | 1.00 | x |  | = | - |
| 6.16. | Procurement and installation of a double-leaf aluminum window, dimensions 197 x 105 cm, with both leaves opening. Material, construction, details, and fittings are to be in accordance with the manufacturer's technology. |  |  |  |  |  |  |
|  | Calculation per unit | unit | 1.00 | x |  | = | - |
| Total carpentry works |  |  |  |  |  |  |  |
| VII | PLASTERING |  |  |  |  |  |  |
| 7.1. | Plastering of internal walls and ceilings in the building. All openings' surfaces have been excluded in the calculation. |  |  |  |  |  |  |
|  | Calculation per m² | m² | 320.00 | x |  | = | - |
|  | Total plastering: |  |  |  |  |  | - |
| VIII | TILING WORKS |  |  |  |  |  |  |
| 8.1. | Procurement and installation of floor tiles in internal spaces, on adhesive, with grout joints aligned. |  |  |  |  |  |  |
|  | Calculation per m² | m² | 140.00 | x |  | = | - |
| 8.2. | Procurement and installation of interior wall tiles, on adhesive, with grout joints aligned. Wall tiles are to be installed from floor to ceiling. |  |  |  |  |  |  |
|  | Calculation per m² | m² | 45.00 | x |  | = | - |
| 8.3. | Procurement and installation of anti-slip floor tiles on the accessible roof of the building and at the staircases. |  |  |  |  |  |  |
|  | Calculation per m² | m² | 155.00 | x |  | = | - |
| 8.4. | Procurement and installation of exterior floor tiles on porches, access staircases, and in the atrium. The tiles must be anti-slip. |  |  |  |  |  |  |
|  | Calculation per m² | m² | 27.00 | x |  | = | - |
| 8.5. | Treatment of the interior side of the parapets with a specific material. |  |  |  |  |  |  |
|  | Calculation per m² | m² | 18.00 | x |  | = | - |
|  | Total tiling works: |  |  |  |  |  | - |

| IX | PAINTING AND DECORATING WORK |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9.1. | Skimming of walls and ceilings |  |  |  |  |  |  |
|  | Calculation per m² | m² | 350.00 | x |  | = | - |
| 9.2. | Painting of interior walls and ceilings with dispersion paint in a tone of choice. |  |  |  |  |  |  |
|  | Calculation per m² | m² | 350.00 | x |  | = | - |
|  | Total painting and decorating work: |  |  |  |  |  | - |
| X | FACADE WORK AND THERMAL INSULATION WORK |  |  |  |  |  |  |
| 10.1. | Procurement and installation of 'demit' facade with thermal insulation made of polystyrene, thickness 5-10 cm. The price includes preparation, procurement, and installation of the facade in a color of choice. |  |  |  |  |  |  |
|  | Calculation per m² | m² | 218.00 | x |  | = | - |
| 10.2. | Procurement and installation of facade details made of perforated metal in a pattern recommended by the designer, with substructure. |  |  |  |  |  |  |
|  | Calculation per m² | m² | 30.00 | x |  | = | - |
|  | Total façade work: |  |  |  |  |  | - |
| XI | METALWORK |  |  |  |  |  |  |
| 11.1. | Procurement and installation of a metal railing for the staircase, in accordance with the project. |  |  |  |  |  |  |
|  | Calculation per m' | m' | 6.00 | x |  | = | - |
| 11.2. | Procurement and installation of a metal railing for the accessible flat roof, in accordance with the project. The height of the railing is 90 cm. |  |  |  |  |  |  |
|  | Calculation per m' | m' | 17.50 | x |  | = | - |
| 11.3. | Procurement and installation of a metal railing for the parapets of the accessible flat roof, in accordance with the project. The height of the railing is 55-60 cm. |  |  |  |  |  |  |
|  | Calculation per m' | m' | 44.00 | x |  | = | - |
|  | Total metalwork: |  |  |  |  |  | - |
| XII | SHEET METAL WORK |  |  |  |  |  |  |
| 12.1. | Fabrication of metal cladding for all parapets of the building's roof. The price includes procurement, fabrication, and installation of these metal elements. |  |  |  |  |  |  |
|  | Calculation per m' | m' | 17.00 | x |  | = | - |
| 12.2. | Installation of vertical downpipes made of coated metal in a color of choice. The price includes procurement and installation of the downpipe. |  |  |  |  |  |  |
|  | Calculation per m' | m' | 16.00 | x |  | = | - |
|  | Total sheet metal work: |  |  |  |  |  | - |
| XIII | PLUMBING |  |  |  |  |  |  |
| 13.1. | Internal plumbing and sewage distribution up to the shafts |  |  |  |  |  |  |
|  | Calculation-flat-rate | flat r. | 1.00 | x |  | = | - |
|  | Total plumbing work: |  |  |  |  |  | - |
| XIV | ELECTRICAL INSTALLATION WORK |  |  |  |  |  |  |
| 14.1. | Internal electrical wiring |  |  |  |  |  |  |
|  | Calculation-flat-rate | flat r. | 1.00 | x |  | = | - |
|  | Total electrical installation work: |  |  |  |  |  | - |

| XV | TELECOMMUNICATION AND SIGNALING INSTALLATION WORK |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15.1. | Internal distribution of telecommunications and signaling installations |  |  |  |  |  |  |
|  | Calculation-flat-rate | flat r. | 1.00 | x |  | = | - |
|  | Total telecommunication and signaling installation work: |  |  |  |  |  | - |
| XVI | PROCUREMENT AND INSTALLATION OF A MOVABLE PERGOLA |  |  |  |  |  |  |
| 16.1. | Procurement and installation of a retractable pergola on the accessible flat roof to create a future seating area with a view of the lake for visitors waiting for their turn to fly in a balloon |  |  |  |  |  |  |
|  | Calculation-flat-rate | flat r. | 1.00 | x |  | = | - |
|  | Total – furniture procurement: |  |  |  |  |  | - |
| XVII | FURNITURE PROCUREMENT |  |  |  |  |  |  |
| 17.1. | Furniture procurement |  |  |  |  |  |  |
|  | Calculation-flat-rate | flat r. | 1.00 | x |  | = | - |
|  | Total - Furniture procurement: |  |  |  |  |  | - |
| XVIII | SUPERVISION DURING THE EXECUTION OF WORK |  |  |  |  |  |  |
| 18.1. | Supervision during the execution of work |  |  |  |  |  |  |
|  | Calculation per number of months of work execution | month | 2.00 | x |  | = | - |
|  | Total- supervision during the execution of work: |  |  |  |  |  | - |
|  | RECAPITULATION: |  |  |  |  |  |  |
| VI | CARPENTRY |  |  |  |  |  | - |
| VII | PLASTERING |  |  |  |  |  | - |
| VIII | TILING |  |  |  |  |  | - |
| IX | PAINTING AND DECORATING WORK |  |  |  |  |  | - |
| X | FACADE WORK |  |  |  |  |  | - |
| XI | METALWORK |  |  |  |  |  |  |
| XII | SHEET METAL WORK |  |  |  |  |  | - |
| XIII | PLUMBING |  |  |  |  |  | - |
| XIV | ELECTRICAL INSTALLATION WORK |  |  |  |  |  | - |
| XV | TELECOMMUNICATION AND SIGNALING INSTALLATION WORK |  |  |  |  |  |  |
| XVI | PROCUREMENT AND INSTALLATION OF A MOVABLE PERGOLA |  |  |  |  |  |  |
| XVII | FURNITURE PROCUREMENT |  |  |  |  |  |  |
| XVIII | SUPERVISION DURING THE EXECUTION OF WORK |  |  |  |  |  |  |
|  | T O T A L (din): |  |  |  |  |  | - |

  - GRAPHIC DOCUMENTATION

GRAPHIC DOCUMENTATION CONTENT

GRAPHIC DOCUMENTATION OF THE EXISTING CONDITION

| 1 | Situation plan |
| --- | --- |
| 2 | Basement plan |
| 3 | Ground floor plan |
| 4 | Accessible flat roof plan |
| 5 | Roof planes plan |
| 6 | Section 1-1 |
| 7 | Section 2-2 |
| 8 | Facade |
| 9 | Facade |
| 10 | Facade |
| 11 | Facade |

GRAPHIC DOCUMENTATION OF THE NEWLY DESIGNED SOLUTION

| 1 | Situation plan |
| --- | --- |
| 2 | Basement plan |
| 3 | Ground floor plan |
| 4 | Accessible flat roof plan |
| 5 | Roof planes plan |
| 6 | Section 1-1 |
| 7 | Section 2-2 |
| 8 | Facade |
| 9 | Facade |
| 10 | Facade |
| 11 | Facade |

GRAPHIC DOCUMENTATION OF THE EXISTING CONDITION

SITE PLAN	S 1:100

-

+

- BUILDING LINE

- REGULATION LINE

- BOUNDARIES OF PLOTS CADASTRAL STATUS

HIGH AND LOW GREENERY

BUSINESS FACILITY

FACILITY ENTRANCE

PARKING SPACE 2.3x4.8m

| UDIO FOR DESIGN OF CONSTRUCTION AND OTHER FACILITIES | RESPONSIBLE DESIGNER: Miodrag Cvetkovic, MSc. In Arch. Eng LICENSE NUMBER: 300 R403 19 | INVESTOR: Municipality Merosina 17 Cara Lazara St., Merosina |  |  | TECH. DOC. MARK: IDR |
| --- | --- | --- | --- | --- | --- |
| 1 - ARCHITECTURE PROJECT |  | FACILITY | Business facility Po+P (basement+ground floor) Cad. Plot. 1358/2, C.M. Oblacina (building No. 5) |  |  |
| RESPONSIBLE Miodrag Cvetkovic DESIGNER. MSc. In Arch. Eng. [Signature] |  | DRAWING NAME:: SITE PLAN |  |  |  |
| ASSOCIATE DESIGNER: |  | DATE: September 2024. |  | SCALE:: S 1:100 | DRAWING No. 1 |

Do not alter the scale of the drawing; use only the stated dimensions. Any discrepancies between this and any other document or agreement must be reported to the chief designer.
This drawing and all the information within it are protected by copyright and are the property of the designer. The net usable area has been calculated according to the SRPS standard U.C2.100: 2002

#### BASEMENT PLAN	S 1:50

LEGEND	BASEMENT

TOTAL		8.79

P net area = 8.79m²

P gross developed construction area = 14.24m²

| UDIO FOR DESIGN OF CONSTRUCTION AND OTHER FACILITIES | RESPONSIBLE DESIGNER: Miodrag Cvetkovic, MSc. In Arch. Eng LICENSE NUMBER: 300 R403 19 | INVESTOR: Municipality Merosina 17 Cara Lazara St., Merosina |  |  | TECH. DOC. MARK: IDR |
| --- | --- | --- | --- | --- | --- |
| 1 - ARCHITECTURE PROJECT |  | FACILITY | Business facility Po+P (basement+ground floor) Cad. Plot. 1358/2, C.M. Oblacina (building No. 5) |  |  |
| RESPONSIBLE Miodrag Cvetkovic DESIGNER. MSc. In Arch. Eng. [Signature] |  | DRAWING NAME:: BASEMENT PLAN |  |  |  |
| ASSOCIATE DESIGNER: |  | DATE: September 2024. |  | SCALE:: S 1:50 | DRAWING No. 2 |

Do not alter the scale of the drawing; use only the stated dimensions. Any discrepancies between this and any other document or agreement must be reported to the chief designer.
This drawing and all the information within it are protected by copyright and are the property of the designer. The net usable area has been calculated according to the SRPS standard U.C2.100: 2002

#### GROUND FLOOR PLAN		S1:50

LEGEND	GROUND FLOOR

TOTAL		143.54

P net area = 143.54m²

P gross developed construction area = 182.00m²

| UDIO FOR DESIGN OF CONSTRUCTION AND OTHER FACILITIES | RESPONSIBLE DESIGNER: Miodrag Cvetkovic, MSc. In Arch. Eng LICENSE NUMBER: 300 R403 19 | INVESTOR: Municipality Merosina 17 Cara Lazara St., Merosina |  |  | TECH. DOC. MARK: IDR |
| --- | --- | --- | --- | --- | --- |
| 1 - ARCHITECTURE PROJECT |  | FACILITY | Business facility Po+P (basement+ground floor) Cad. Plot. 1358/2, C.M. Oblacina (building No. 5) |  |  |
| RESPONSIBLE Miodrag Cvetkovic DESIGNER. MSc. In Arch. Eng. [Signature] |  | DRAWING NAME:: GROUND FLOOR PLAN |  |  |  |
| ASSOCIATE DESIGNER: |  | DATE: September 2024. |  | SCALE:: S 1:50 | DRAWING No. 3 |

Do not alter the scale of the drawing; use only the stated dimensions. Any discrepancies between this and any other document or agreement must be reported to the chief designer.
	This drawing and all the information within it are protected by copyright and are the property of the designer. The net usable area has been calculated according to the SRPS standard U.C2.100: 2002

WALKABLE ROOF PLAN	S 1:50

I

LEGEND

| UDIO FOR DESIGN OF CONSTRUCTION AND OTHER FACILITIES | RESPONSIBLE DESIGNER: Miodrag Cvetkovic, MSc. In Arch. Eng LICENSE NUMBER: 300 R403 19 | INVESTOR: Municipality Merosina 17 Cara Lazara St., Merosina |  |  | TECH. DOC. MARK: IDR |
| --- | --- | --- | --- | --- | --- |
| 1 - ARCHITECTURE PROJECT |  | FACILITY | Business facility Po+P (basement+ground floor) Cad. Plot. 1358/2, C.M. Oblacina (building No. 5) |  |  |
| RESPONSIBLE Miodrag Cvetkovic DESIGNER. MSc. In Arch. Eng. [Signature] |  | DRAWING NAME:: WALKABLE ROOF PLAN |  |  |  |
| ASSOCIATE DESIGNER: |  | DATE: September 2024. |  | SCALE:: S 1:50 | DRAWING No. 4 |

Do not alter the scale of the drawing; use only the stated dimensions. Any discrepancies between this and any other document or agreement must be reported to the chief designer.
	This drawing and all the information within it are protected by copyright and are the property of the designer. The net usable area has been calculated according to the SRPS standard U.C2.100: 2002

#### ROOF PLANES PLAN 1-1	S 1:50

| UDIO FOR DESIGN OF CONSTRUCTION AND OTHER FACILITIES | RESPONSIBLE DESIGNER: Miodrag Cvetkovic, MSc. In Arch. Eng LICENSE NUMBER: 300 R403 19 | INVESTOR: Municipality Merosina 17 Cara Lazara St., Merosina |  |  | TECH. DOC. MARK: IDR |
| --- | --- | --- | --- | --- | --- |
| 1 - ARCHITECTURE PROJECT |  | FACILITY | Business facility Po+P (basement+ground floor) Cad. Plot. 1358/2, C.M. Oblacina (building No. 5) |  |  |
| RESPONSIBLE Miodrag Cvetkovic DESIGNER. MSc. In Arch. Eng. [Signature] |  | DRAWING NAME:: ROOF PLANES PLAN |  |  |  |
| ASSOCIATE DESIGNER: |  | DATE: September 2024. |  | SCALE:: S 1:50 | DRAWING No. 5 |

Do not alter the scale of the drawing; use only the stated dimensions. Any discrepancies between this and any other document or agreement must be reported to the chief designer.
		This drawing and all the information within it are protected by copyright and are the property of the designer. The net usable area has been calculated according to the SRPS standard U.C2.100: 2002

#### SECTION 1-1		S 1:50

| UDIO FOR DESIGN OF CONSTRUCTION AND OTHER FACILITIES | RESPONSIBLE DESIGNER: Miodrag Cvetkovic, MSc. In Arch. Eng LICENSE NUMBER: 300 R403 19 | INVESTOR: Municipality Merosina 17 Cara Lazara St., Merosina |  |  | TECH. DOC. MARK: IDR |
| --- | --- | --- | --- | --- | --- |
| 1 - ARCHITECTURE PROJECT |  | FACILITY | Business facility Po+P (basement+ground floor) Cad. Plot. 1358/2, C.M. Oblacina (building No. 5) |  |  |
| RESPONSIBLE Miodrag Cvetkovic DESIGNER. MSc. In Arch. Eng. [Signature] |  | DRAWING NAME:: SECTION 1:1 |  |  |  |
| ASSOCIATE DESIGNER: |  | DATE: September 2024. |  | SCALE:: S 1:50 | DRAWING No. 6 |

Do not alter the scale of the drawing; use only the stated dimensions. Any discrepancies between this and any other document or agreement must be reported to the chief designer.
		This drawing and all the information within it are protected by copyright and are the property of the designer. The net usable area has been calculated according to the SRPS standard U.C2.100: 2002

#### SECTION 2-2		S 1:50

| UDIO FOR DESIGN OF CONSTRUCTION AND OTHER FACILITIES | RESPONSIBLE DESIGNER: Miodrag Cvetkovic, MSc. In Arch. Eng LICENSE NUMBER: 300 R403 19 | INVESTOR: Municipality Merosina 17 Cara Lazara St., Merosina |  |  | TECH. DOC. MARK: IDR |
| --- | --- | --- | --- | --- | --- |
| 1 - ARCHITECTURE PROJECT |  | FACILITY | Business facility Po+P (basement+ground floor) Cad. Plot. 1358/2, C.M. Oblacina (building No. 5) |  |  |
| RESPONSIBLE Miodrag Cvetkovic DESIGNER. MSc. In Arch. Eng. [Signature] |  | DRAWING NAME:: SECTION 2:2 |  |  |  |
| ASSOCIATE DESIGNER: |  | DATE: September 2024. |  | SCALE:: S 1:50 | DRAWING No. 7 |

Do not alter the scale of the drawing; use only the stated dimensions. Any discrepancies between this and any other document or agreement must be reported to the chief designer.
		This drawing and all the information within it are protected by copyright and are the property of the designer. The net usable area has been calculated according to the SRPS standard U.C2.100: 2002

#### SOUTHWEST FACADE	S 1:50

| UDIO FOR DESIGN OF CONSTRUCTION AND OTHER FACILITIES | RESPONSIBLE DESIGNER: Miodrag Cvetkovic, MSc. In Arch. Eng LICENSE NUMBER: 300 R403 19 | INVESTOR: Municipality Merosina 17 Cara Lazara St., Merosina |  |  | TECH. DOC. MARK: IDR |
| --- | --- | --- | --- | --- | --- |
| 1 - ARCHITECTURE PROJECT |  | FACILITY | Business facility Po+P (basement+ground floor) Cad. Plot. 1358/2, C.M. Oblacina (building No. 5) |  |  |
| RESPONSIBLE Miodrag Cvetkovic DESIGNER. MSc. In Arch. Eng. [Signature] |  | DRAWING NAME:: SOUTHWEST FACADE |  |  |  |
| ASSOCIATE DESIGNER: |  | DATE: September 2024. |  | SCALE:: S 1:50 | DRAWING No. 8 |

Do not alter the scale of the drawing; use only the stated dimensions. Any discrepancies between this and any other document or agreement must be reported to the chief designer.
		This drawing and all the information within it are protected by copyright and are the property of the designer. The net usable area has been calculated according to the SRPS standard U.C2.100: 2002

#### SOUTHEAST FACADE	S 1:50

| UDIO FOR DESIGN OF CONSTRUCTION AND OTHER FACILITIES | RESPONSIBLE DESIGNER: Miodrag Cvetkovic, MSc. In Arch. Eng LICENSE NUMBER: 300 R403 19 | INVESTOR: Municipality Merosina 17 Cara Lazara St., Merosina |  |  | TECH. DOC. MARK: IDR |
| --- | --- | --- | --- | --- | --- |
| 1 - ARCHITECTURE PROJECT |  | FACILITY | Business facility Po+P (basement+ground floor) Cad. Plot. 1358/2, C.M. Oblacina (building No. 5) |  |  |
| RESPONSIBLE Miodrag Cvetkovic DESIGNER. MSc. In Arch. Eng. [Signature] |  | DRAWING NAME:: SOUTHEAST FACADE |  |  |  |
| ASSOCIATE DESIGNER: |  | DATE: September 2024. |  | SCALE:: S 1:50 | DRAWING No. 9 |

Do not alter the scale of the drawing; use only the stated dimensions. Any discrepancies between this and any other document or agreement must be reported to the chief designer.
		This drawing and all the information within it are protected by copyright and are the property of the designer. The net usable area has been calculated according to the SRPS standard U.C2.100: 2002

-

NORTHEAST FACADE	S 1:50

| UDIO FOR DESIGN OF CONSTRUCTION AND OTHER FACILITIES | RESPONSIBLE DESIGNER: Miodrag Cvetkovic, MSc. In Arch. Eng LICENSE NUMBER: 300 R403 19 | INVESTOR: Municipality Merosina 17 Cara Lazara St., Merosina |  |  | TECH. DOC. MARK: IDR |
| --- | --- | --- | --- | --- | --- |
| 1 - ARCHITECTURE PROJECT |  | FACILITY | Business facility Po+P (basement+ground floor) Cad. Plot. 1358/2, C.M. Oblacina (building No. 5) |  |  |
| RESPONSIBLE Miodrag Cvetkovic DESIGNER. MSc. In Arch. Eng. [Signature] |  | DRAWING NAME:: NORTHEAST FACADE |  |  |  |
| ASSOCIATE DESIGNER: |  | DATE: September 2024. |  | SCALE:: S 1:50 | DRAWING No. 10 |

Do not alter the scale of the drawing; use only the stated dimensions. Any discrepancies between this and any other document or agreement must be reported to the chief designer.
		This drawing and all the information within it are protected by copyright and are the property of the designer. The net usable area has been calculated according to the SRPS standard U.C2.100: 2002

-

NORTHWEST FACADE	S 1:50

| UDIO FOR DESIGN OF CONSTRUCTION AND OTHER FACILITIES | RESPONSIBLE DESIGNER: Miodrag Cvetkovic, MSc. In Arch. Eng LICENSE NUMBER: 300 R403 19 | INVESTOR: Municipality Merosina 17 Cara Lazara St., Merosina |  |  | TECH. DOC. MARK: IDR |
| --- | --- | --- | --- | --- | --- |
| 1 - ARCHITECTURE PROJECT |  | FACILITY | Business facility Po+P (basement+ground floor) Cad. Plot. 1358/2, C.M. Oblacina (building No. 5) |  |  |
| RESPONSIBLE Miodrag Cvetkovic DESIGNER. MSc. In Arch. Eng. [Signature] |  | DRAWING NAME:: NORTHWEST FACADE |  |  |  |
| ASSOCIATE DESIGNER: |  | DATE: September 2024. |  | SCALE:: S 1:50 | DRAWING No. 11 |

Do not alter the scale of the drawing; use only the stated dimensions. Any discrepancies between this and any other document or agreement must be reported to the chief designer.
		This drawing and all the information within it are protected by copyright and are the property of the designer. The net usable area has been calculated according to the SRPS standard U.C2.100: 2002

u

1.7.2	GRAPHIC DOCUMENTATION OF THE NEWLY DESIGNED SOLUTION

SITE PLAN	S 1:100

-

+

- BUILDING LINE

- REGULATION LINE

- BOUNDARIES OF PLOTS CADASTRAL STATUS

HIGH AND LOW GREENERY

BUSINESS FACILITY

FACILITY ENTRANCE

PARKING SPACE 2.3x4.8m

| DIO FOR DESIGN OF CONSTRUCTION AND OTHER FACILITIES | RESPONSIBLE DESIGNER: Miodrag Cvetkovic, MSc. In Arch. Eng LICENSE NUMBER: 300 R403 19 | INVESTOR: Municipality Merosina 17 Cara Lazara St., Merosina |  |  | TECH. DOC. MARK: IDR |
| --- | --- | --- | --- | --- | --- |
| 1 - ARCHITECTURE PROJECT |  | FACILITY | Business facility Po+P (basement+ground floor) Cad. Plot. 1358/2, C.M. Oblacina (building No. 5) |  |  |
| RESPONSIBLE Miodrag Cvetkovic DESIGNER. MSc. In Arch. Eng. [Signature] |  | DRAWING NAME:: SITE PLAN |  |  |  |
| ASSOCIATE DESIGNER: |  | DATE: September 2024. |  | SCALE:: S 1:100 | DRAWING No. 1 |

Do not alter the scale of the drawing; use only the stated dimensions. Any discrepancies between this and any other document or agreement must be reported to the chief designer.
This drawing and all the information within it are protected by copyright and are the property of the designer. The net usable area has been calculated according to the SRPS standard U.C2.100: 2002

#### BASEMENT PLAN	S 1:50

LEGEND	BASEMENT

TOTAL		8.79

P net area = 8.79m²

P gross developed construction area = 14.24m²

| UDIO FOR DESIGN OF CONSTRUCTION AND OTHER FACILITIES | RESPONSIBLE DESIGNER: Miodrag Cvetkovic, MSc. In Arch. Eng LICENSE NUMBER: 300 R403 19 | INVESTOR: Municipality Merosina 17 Cara Lazara St., Merosina |  |  | TECH. DOC. MARK: IDR |
| --- | --- | --- | --- | --- | --- |
| 1 - ARCHITECTURE PROJECT |  | FACILITY | Business facility Po+P (basement+ground floor) Cad. Plot. 1358/2, C.M. Oblacina (building No. 5) |  |  |
| RESPONSIBLE Miodrag Cvetkovic DESIGNER. MSc. In Arch. Eng. [Signature] |  | DRAWING NAME:: BASEMENT PLAN |  |  |  |
| ASSOCIATE DESIGNER: |  | DATE: September 2024. |  | SCALE:: S 1:50 | DRAWING No. 2 |

Do not alter the scale of the drawing; use only the stated dimensions. Any discrepancies between this and any other document or agreement must be reported to the chief designer.
This drawing and all the information within it are protected by copyright and are the property of the designer. The net usable area has been calculated according to the SRPS standard U.C2.100: 2002

#### GROUND FLOOR PLAN		S1:50

LEGEND OF THE GROUND FLOOR

TOTAL 		144.03

P net area = 144.03m²

P gross developed construction area = 182.00m²

| UDIO FOR DESIGN OF CONSTRUCTION AND OTHER FACILITIES | RESPONSIBLE DESIGNER: Miodrag Cvetkovic, MSc. In Arch. Eng LICENSE NUMBER: 300 R403 19 | INVESTOR: Municipality Merosina 17 Cara Lazara St., Merosina |  |  | TECH. DOC. MARK: IDR |
| --- | --- | --- | --- | --- | --- |
| 1 - ARCHITECTURE PROJECT |  | FACILITY | Business facility Po+P (basement+ground floor) Cad. Plot. 1358/2, C.M. Oblacina (building No. 5) |  |  |
| RESPONSIBLE Miodrag Cvetkovic DESIGNER. MSc. In Arch. Eng. [Signature] |  | DRAWING NAME:: GROUND FLOOR PLAN |  |  |  |
| ASSOCIATE DESIGNER: |  | DATE: September 2024. |  | SCALE:: S 1:50 | DRAWING No. 3 |

Do not alter the scale of the drawing; use only the stated dimensions. Any discrepancies between this and any other document or agreement must be reported to the chief designer.
	This drawing and all the information within it are protected by copyright and are the property of the designer. The net usable area has been calculated according to the SRPS standard U.C2.100: 2002

WALKABLE ROOF PLAN	S 1:50

I

LEGEND

| UDIO FOR DESIGN OF CONSTRUCTION AND OTHER FACILITIES | RESPONSIBLE DESIGNER: Miodrag Cvetkovic, MSc. In Arch. Eng LICENSE NUMBER: 300 R403 19 | INVESTOR: Municipality Merosina 17 Cara Lazara St., Merosina |  |  | TECH. DOC. MARK: IDR |
| --- | --- | --- | --- | --- | --- |
| 1 - ARCHITECTURE PROJECT |  | FACILITY | Business facility Po+P (basement+ground floor) Cad. Plot. 1358/2, C.M. Oblacina (building No. 5) |  |  |
| RESPONSIBLE Miodrag Cvetkovic DESIGNER. MSc. In Arch. Eng. [Signature] |  | DRAWING NAME:: WALKABLE ROOF PLAN |  |  |  |
| ASSOCIATE DESIGNER: |  | DATE: September 2024. |  | SCALE:: S 1:50 | DRAWING No. 4 |

Do not alter the scale of the drawing; use only the stated dimensions. Any discrepancies between this and any other document or agreement must be reported to the chief designer.
	This drawing and all the information within it are protected by copyright and are the property of the designer. The net usable area has been calculated according to the SRPS standard U.C2.100: 2002

#### ROOF PLANES PLAN 1-1	S 1:50

| UDIO FOR DESIGN OF CONSTRUCTION AND OTHER FACILITIES | RESPONSIBLE DESIGNER: Miodrag Cvetkovic, MSc. In Arch. Eng LICENSE NUMBER: 300 R403 19 | INVESTOR: Municipality Merosina 17 Cara Lazara St., Merosina |  |  | TECH. DOC. MARK: IDR |
| --- | --- | --- | --- | --- | --- |
| 1 - ARCHITECTURE PROJECT |  | FACILITY | Business facility Po+P (basement+ground floor) Cad. Plot. 1358/2, C.M. Oblacina (building No. 5) |  |  |
| RESPONSIBLE Miodrag Cvetkovic DESIGNER. MSc. In Arch. Eng. [Signature] |  | DRAWING NAME:: ROOF PLANES PLAN |  |  |  |
| ASSOCIATE DESIGNER: |  | DATE: September 2024. |  | SCALE:: S 1:50 | DRAWING No. 5 |

Do not alter the scale of the drawing; use only the stated dimensions. Any discrepancies between this and any other document or agreement must be reported to the chief designer.
		This drawing and all the information within it are protected by copyright and are the property of the designer. The net usable area has been calculated according to the SRPS standard U.C2.100: 2002

#### SECTION 1-1		S 1:50

| UDIO FOR DESIGN OF CONSTRUCTION AND OTHER FACILITIES | RESPONSIBLE DESIGNER: Miodrag Cvetkovic, MSc. In Arch. Eng LICENSE NUMBER: 300 R403 19 | INVESTOR: Municipality Merosina 17 Cara Lazara St., Merosina |  |  | TECH. DOC. MARK: IDR |
| --- | --- | --- | --- | --- | --- |
| 1 - ARCHITECTURE PROJECT |  | FACILITY | Business facility Po+P (basement+ground floor) Cad. Plot. 1358/2, C.M. Oblacina (building No. 5) |  |  |
| RESPONSIBLE Miodrag Cvetkovic DESIGNER. MSc. In Arch. Eng. [Signature] |  | DRAWING NAME:: SECTION 1:1 |  |  |  |
| ASSOCIATE DESIGNER: |  | DATE: September 2024. |  | SCALE:: S 1:50 | DRAWING No. 6 |

Do not alter the scale of the drawing; use only the stated dimensions. Any discrepancies between this and any other document or agreement must be reported to the chief designer.
		This drawing and all the information within it are protected by copyright and are the property of the designer. The net usable area has been calculated according to the SRPS standard U.C2.100: 2002

#### SECTION 2-2		S 1:50

| UDIO FOR DESIGN OF CONSTRUCTION AND OTHER FACILITIES | RESPONSIBLE DESIGNER: Miodrag Cvetkovic, MSc. In Arch. Eng LICENSE NUMBER: 300 R403 19 | INVESTOR: Municipality Merosina 17 Cara Lazara St., Merosina |  |  | TECH. DOC. MARK: IDR |
| --- | --- | --- | --- | --- | --- |
| 1 - ARCHITECTURE PROJECT |  | FACILITY | Business facility Po+P (basement+ground floor) Cad. Plot. 1358/2, C.M. Oblacina (building No. 5) |  |  |
| RESPONSIBLE Miodrag Cvetkovic DESIGNER. MSc. In Arch. Eng. [Signature] |  | DRAWING NAME:: SECTION 2:2 |  |  |  |
| ASSOCIATE DESIGNER: |  | DATE: September 2024. |  | SCALE:: S 1:50 | DRAWING No. 7 |

Do not alter the scale of the drawing; use only the stated dimensions. Any discrepancies between this and any other document or agreement must be reported to the chief designer.
		This drawing and all the information within it are protected by copyright and are the property of the designer. The net usable area has been calculated according to the SRPS standard U.C2.100: 2002

#### SOUTHWEST FACADE	S 1:50

| UDIO FOR DESIGN OF CONSTRUCTION AND OTHER FACILITIES | RESPONSIBLE DESIGNER: Miodrag Cvetkovic, MSc. In Arch. Eng LICENSE NUMBER: 300 R403 19 | INVESTOR: Municipality Merosina 17 Cara Lazara St., Merosina |  |  | TECH. DOC. MARK: IDR |
| --- | --- | --- | --- | --- | --- |
| 1 - ARCHITECTURE PROJECT |  | FACILITY | Business facility Po+P (basement+ground floor) Cad. Plot. 1358/2, C.M. Oblacina (building No. 5) |  |  |
| RESPONSIBLE Miodrag Cvetkovic DESIGNER. MSc. In Arch. Eng. [Signature] |  | DRAWING NAME:: SOUTHWEST FACADE |  |  |  |
| ASSOCIATE DESIGNER: |  | DATE: September 2024. |  | SCALE:: S 1:50 | DRAWING No. 8 |

Do not alter the scale of the drawing; use only the stated dimensions. Any discrepancies between this and any other document or agreement must be reported to the chief designer.
		This drawing and all the information within it are protected by copyright and are the property of the designer. The net usable area has been calculated according to the SRPS standard U.C2.100: 2002

#### SOUTHEAST FACADE	S 1:50

| UDIO FOR DESIGN OF CONSTRUCTION AND OTHER FACILITIES | RESPONSIBLE DESIGNER: Miodrag Cvetkovic, MSc. In Arch. Eng LICENSE NUMBER: 300 R403 19 | INVESTOR: Municipality Merosina 17 Cara Lazara St., Merosina |  |  | TECH. DOC. MARK: IDR |
| --- | --- | --- | --- | --- | --- |
| 1 - ARCHITECTURE PROJECT |  | FACILITY | Business facility Po+P (basement+ground floor) Cad. Plot. 1358/2, C.M. Oblacina (building No. 5) |  |  |
| RESPONSIBLE Miodrag Cvetkovic DESIGNER. MSc. In Arch. Eng. [Signature] |  | DRAWING NAME:: SOUTHEAST FACADE |  |  |  |
| ASSOCIATE DESIGNER: |  | DATE: September 2024. |  | SCALE:: S 1:50 | DRAWING No. 9 |

Do not alter the scale of the drawing; use only the stated dimensions. Any discrepancies between this and any other document or agreement must be reported to the chief designer.
		This drawing and all the information within it are protected by copyright and are the property of the designer. The net usable area has been calculated according to the SRPS standard U.C2.100: 2002

#### NORTHEAST FACADE	S 1:50

| UDIO FOR DESIGN OF CONSTRUCTION AND OTHER FACILITIES | RESPONSIBLE DESIGNER: Miodrag Cvetkovic, MSc. In Arch. Eng LICENSE NUMBER: 300 R403 19 | INVESTOR: Municipality Merosina 17 Cara Lazara St., Merosina |  |  | TECH. DOC. MARK: IDR |
| --- | --- | --- | --- | --- | --- |
| 1 - ARCHITECTURE PROJECT |  | FACILITY | Business facility Po+P (basement+ground floor) Cad. Plot. 1358/2, C.M. Oblacina (building No. 5) |  |  |
| RESPONSIBLE Miodrag Cvetkovic DESIGNER. MSc. In Arch. Eng. [Signature] |  | DRAWING NAME:: NORTHEAST FACADE |  |  |  |
| ASSOCIATE DESIGNER: |  | DATE: September 2024. |  | SCALE:: S 1:50 | DRAWING No. 10 |

Do not alter the scale of the drawing; use only the stated dimensions. Any discrepancies between this and any other document or agreement must be reported to the chief designer.
		This drawing and all the information within it are protected by copyright and are the property of the designer. The net usable area has been calculated according to the SRPS standard U.C2.100: 2002

#### NORTHWEST FACADE	S 1:50

| UDIO FOR DESIGN OF CONSTRUCTION AND OTHER FACILITIES | RESPONSIBLE DESIGNER: Miodrag Cvetkovic, MSc. In Arch. Eng LICENSE NUMBER: 300 R403 19 | INVESTOR: Municipality Merosina 17 Cara Lazara St., Merosina |  |  | TECH. DOC. MARK: IDR |
| --- | --- | --- | --- | --- | --- |
| 1 - ARCHITECTURE PROJECT |  | FACILITY | Business facility Po+P (basement+ground floor) Cad. Plot. 1358/2, C.M. Oblacina (building No. 5) |  |  |
| RESPONSIBLE Miodrag Cvetkovic DESIGNER. MSc. In Arch. Eng. [Signature] |  | DRAWING NAME:: NORTHWEST FACADE |  |  |  |
| ASSOCIATE DESIGNER: |  | DATE: September 2024. |  | SCALE:: S 1:50 | DRAWING No. 11 |

Do not alter the scale of the drawing; use only the stated dimensions. Any discrepancies between this and any other document or agreement must be reported to the chief designer.
		This drawing and all the information within it are protected by copyright and are the property of the designer. The net usable area has been calculated according to the SRPS standard U.C2.100: 200